6-DOF ZWHAND Interface Function Description (Python)

V1.0

Document Revision History：

| Version | Date | Remarks |
| --- | --- | --- |
| V1.0 | 2026.4.1 | Draft |

Tables of Contents

Python Syntax Overview	4

(1) Example Code Description	7

(2) Interface Function Set	8

(3) Interface Function Examples	11

1、 Get ZWHAND Device Initialization Status	11

2、 Set ZWHAND Device ID	12

3、 Set the position of the full hand joint motor of ZWHAND	13

4、 Set the full hand joint motor PVT control mode of ZWHAND	14

5、 Set the full hand joint motor PC control mode of ZWHAND	16

Python Syntax Overview

Python is a popular high-level programming language. It is easy to learn and read, and it features a rich ecosystem of libraries and tools. Below are some basic Python syntax concepts:

1. Comments：Single-line comments are indicated using the # symbol. Comments are used to explain code logic and provide documentation.

- # This is a single-line comment.

Multi-line comments can be written using triple quotes (single or double quotes):

- '''

- This is a

- multi-line comment

- '''

2. Indentation：Python uses indentation to define code blocks, rather than braces or other symbols. Typically, four spaces are used for one indentation level.

- if True:

- print("This is an indented code block")

3. Variables：Variables are used to store data. There is no need to explicitly declare variable types, as Python automatically infers them. Variable names are case-sensitive.

- x = 10

- name = "zwhand"

4. Data Types：Python supports a variety of data types, including integers (int), floating-point numbers (float), strings (str), booleans (bool), lists (list), tuples (tuple), sets (set), and dictionaries (dict).

- message = "Hello, World!"

- is_true = True

- my_list = [1, 2, 3]

- my_tuple = (4, 5, 6)

- my_set = {7, 8, 9}

- my_dict = {"name": "zwhand", "age": 20}

5. Operators：Python supports various operators, including arithmetic operators (+, -, *, /, %), comparison operators (==, !=, <, >), and logical operators (and, or, not).

- x = 5

- y = 3

- result = x + y # addition

- is_equal = x == y # Equality comparison

- is_greater = x > y # Greater than comparison

- logical_result = (x > 0) and (y < 11) # Logic and Operation

6. Conditional Statements：Conditional branching is implemented using “if”, “elif” (optional), and “else” (optional).

- if condition:

- # If the condition is true, execute the code here

- elif another_condition:

- # If the above condition is not met, and this condition is true, execute the code here

- else:

- # If none of the above conditions are met, execute the code here

7. Loops：Python supports “for” loops and “while” loops.

- for i in range(5): # Use the range function to generate a set of numbers

- print(i)

- while condition:

- # If the condition is true, execute the code here

8. Functions：Functions are defined using the “def” keyword.

- def greet(name):

- print("Hello, " + name)

- greet("zwhand")

9. List Comprehensions：List comprehensions provide a concise way to create lists.

- squares = [x**2 for x in range(6)]

10. Exception Handling：Use "try" and "except" to catch and handle exceptions.

- try:

- result = 17 / 0

- except ZeroDivisionError:

- print("除以零错误")

This section only covers basic Python syntax. For a deeper understanding of Python, please refer to the official Python documentation or more comprehensive tutorials and learning resources.

- Example Code Description

The file “DOF6_ZWHAND_API.py” provides the core functional interface for controlling the ZWHAND device.

- This document serves as a Python secondary development interface description based on the Modbus protocol.

- Supported Platforms：Linux、Windows

- Python Version：python 3.9 or later

- Open-Source Libraries： pyserial

- Interface Function Set

| Function Name | Interface Definition | Parameter Description |
| --- | --- | --- |
| ZWHAND() | The interface class of ZWHAND | - Port Number 2、Baud Rate（default：115200） 3、ZWHAND ID（range：0x01-0xFF） |
| open_start_device() | Open device | None |
| close_device() | Close device | None |
| send_single_data() | Send single data | 1、Register Address 2、Data（range：0x0000-0xFFFF） 3、The sign bit of the data（default：unsigned） |
| send_multiple_data() | Send multiple data | 1、Register Starting Address 2、Data List 3、The sign bit of the data（default：unsigned） |
| read_multiple_data() | Read multiple data | 1、Register Starting Address 2、Data Length 3、The sign bit of the data（default：signed） |
| set_id() | Set ID | 1、New ID（range：0x01-0xFF） |
| set_baud() | Set the baud rate | 1、Baud Rate Level（range：0x01-0x04） Level description：0x01 - '9600' 0x02 - '115200' 0x03 - '921600' 0x04 - '2000000' |
| set_error_clear() | Clear the fault | None |
| set_power_off_save() | Set for power-off data saving | 1、Save Parameters（range：0x01-0x02） Parameter description：0x01 - Configuration saved(ID、baud rate)；0x02 - Parameter saved(speed、current) |
| set_factory_data_reset() | Factory data reset | None |
| set_single_motor_speed() | Set the speed of a single joint motor | 1、Joint Number（range：0x01-0x06） 2、Speed Level（range：1- 100） |
| set_all_motor_speed() | Set the speeds of all joint motors | 1、List of speed levels（length：0x06） Examples：[100, 100, 100, 100, 100, 100] |
| set_single_motor_current() | Set the current of a single joint motor | 1、Joint Number（range：0x01-0x06） 2、Current Level（range：1- 100） |
| set_all_motor_current() | Set the current of all joint motors | 1、List of current levels（length：0x06） Examples：[50, 50, 50, 50, 50, 50] |
| set_single_motor_stop() | Disable a single joint motor | 1、Joint Number（range：0x01-0x06） |
| set_all_motor_stop() | Disable all joint motors | None |
| set_single_motor_absolute() | Set the absolute position of a single joint motor | 1、Joint Number（range：0x01-0x06） 2、Absolute Position（range：0 - 1000） Note: Only the PSC control mode is in effect. |
| set_all_motor_absolute() | Set the absolute positions of all joint motors | 1、Absolute Position List（length：0x06） Examples：[0, 500,1000, 1000, 1000, 1000] Note: Only the PSC control mode is in effect. |
| set_single_motor_relative() | Set the relative position of a single joint motor | 1、Joint Number（range：0x01-0x06） 2、Relative Position（range：-1000 - 1000） Note: Only the PSC control mode is in effect. |
| set_all_motor_relative() | Set the relative positions of all joint motors | 1、Relative Position List（length：0x06） Examples：[0, 500, 1000, 0, -500, -1000] Note: Only the PSC control mode is in effect. |
| set_single_motor_calibration() | Set the calibration for a single joint motor | 1、Joint Number（range：0x01-0x06） |
| set_all_motor_calibration() | Set calibration for all joint motors | None |
| set_all_motor_radian() | Set the angles of all joint motors | 1、List of joint angles（length：0x06；range：0-max radian） Examples：[0, 500, 1000, 1800, 2200, 2600] Note: Only the PSC control mode is in effect. |
| set_finger_control_mode() | Set the control mode of the ZWHAND | 1、Control Mode（range：0x0-0x02） Parameter description： 0x00 - Current Control Mode（PC） 0x01 - Position Control Mode（PSC） 0x02 - Position - Velocity - Time Mode（PVT） |
| set_pvt_control_param() | Set the PVT control parameters for all joint motors | PVT Control Parameters（length：3*0x06） Parameter description：V非0，T为0 Format：[P, V, T] *0x06 Examples：[500, 33000, 0] *0x06 range：P（0-1000） V（0-33000） T（450-5000） Note: Only the PVT control mode is in effect. |
| set_pc_control_param() | Set the control parameters for all joint motors using the PC | PC Control Parameters（length：0x06；range：-8000-8000） Examples：[-8000, -4000, 5000, 6000, 7000, 8000] Note: Only the PC control mode is in effect. |
| get_initialize_state() | Obtain the initialization status of the device | None Return：Initialization State |
| get_bootloader_version() | ‌Get the version of the Boot Loader | None Return：Boot Loader Version |
| get_hardware_version() | ‌Get the hardware version | None Return：Hardware Version |
| get_software_version() | ‌Get the software version | None Return：Software Version |
| get_device_error() | ‌Get the Hall fault status of all joint motors | None Return：List of Hall Fault States [Motor 1, ...... ,Motor6] |
| get_device_voltage() | ‌Get the voltage of the device | None Return：Device Voltage（V） |
| get_motor_locked_state() | ‌Get the blocked-rotation status of all joint motors | None Return：List of motor locked-rotor status [Motor 1, ...... ,Motor6] |
| get_motor_real_angle() | ‌Get the positions of all joint motors | None Return：Motor Position List [Motor 1, ...... ,Motor6] |
| get_all_motor_speed() | ‌Get the speeds of all joint motors | None Return：Motor Speed List [Motor 1, ...... ,Motor6] |
| get_all_motor_current() | ‌Get the current of all joint motors | None Return：Motor Current List [Motor 1, ...... ,Motor6] |
| get_phase_loss_fault() | ‌Get the fault status of all joint motors that are in a phase failure state | None Return：List of motor phase failure states [Motor 1, ...... ,Motor6] |
| get_cur_samp_fault() | ‌Get all joint motor current sampling failure | None Return：List of motor current sampling failure states [Motor 1, ...... ,Motor6] |
| get_all_motor_radian() | ‌Get the angles of all joint motors | None Return：Motor Angle List [Motor 1, ...... ,Motor6] |
| get_all_motor_max_radian() | ‌Get the maximum angular values that all joint motors can move to | None Return：List of maximum angular movements of the motor [Motor 1, ...... ,Motor6] |
| get_cust_number() | ‌Get the customer number | None Return：Customer Number |
| get_ele_skin_force_infor() | ‌Get the force data of the electronic skin | None Return：List of force data of the electronic skin [Little, Ring, Middle, Index, Thumb, Palm] |
| get_all_motor_temperature() | ‌Get the temperature of all joint motors | None Return：Motor Temperature List [Motor 1, ...... ,Motor6] |

- Interface Function Examples

- Get ZWHAND Device Initialization Status

- from DOF6_ZWHAND_API import ZWHAND


if __name__ == '__main__':
    hand = ZWHAND('COM1', 115200, 0x01)
    open_ret = hand.open_start_device()
    if open_ret:
        print("Device open successful!")
        init_state = hand.get_initialize_state()
        if init_state:
            print("Device initialization successful!")
        else:
            print("Device initialization failed!")
    else:
        print("Device open failed!")

- 结果输出：

- Device open successful!

- 发送数据: 01 04 00 00 00 01 31 CA

- 接收数据: 01 04 02 00 01 78 F0

- Device initialization successful!

- Set ZWHAND Device ID

- from DOF6_ZWHAND_API import ZWHAND


if __name__ == '__main__':
    hand = ZWHAND('COM1', 115200, 0x01)
    open_ret = hand.open_start_device()
    if open_ret:
        print("Device open successful!")
        init_state = hand.get_initialize_state()
        if init_state:
            print("Device initialization successful!")
            set_ret = hand.set_id(0x01)
            if set_ret:
                print("Set id successful!")
                save_ret = hand.set_power_off_save(0x01)
                if save_ret:
                    print("Set power off save successful!")
                else:
                    print("Set power off save failed!")
            else:
                print("Set id failed!")
        else:
            print("Device initialization failed!")
        close_ret = hand.close_device()
        if close_ret:
            print("Device close successful!")
        else:
            print("Device close failed!")
    else:
        print("Device open failed!")

- 结果输出：

- Device open successful!

- 发送数据: 01 04 00 00 00 01 31 CA

- 接收数据: 01 04 02 00 01 78 F0

- Device initialization successful!

- 发送数据: 01 06 00 00 00 01 48 0A

- 接收数据: 01 06 00 00 00 01 48 0A

- Set id successful!

- 发送数据: 01 06 00 03 00 01 B8 0A

- 接收数据: 01 06 00 03 00 01 B8 0A

- Set power off save successful!

- Device close successful!

- Set the position of the full hand joint motor of ZWHAND

- from DOF6_ZWHAND_API import ZWHAND


if __name__ == '__main__':
    hand = ZWHAND('COM1', 115200, 0x01)
    open_ret = hand.open_start_device()
    if open_ret:
        print("Device open successful!")
        init_state = hand.get_initialize_state()
        if init_state:
            print("Device initialization successful!")
            set_speed_ret = hand.set_all_motor_speed([100]*0x06)
            if set_speed_ret:
                print("Motor set speed successful!")
            else:
                print("Motor set speed failed!")
            calibration_ret = hand.set_all_motor_calibration()
            if calibration_ret:
                print("Motor calibration successful!")
            else:
                print("Motor calibration failed!")
            time.sleep(20)
            set_position_ret = hand.set_all_motor_absolute([0, 500, 1000, 1000, 1000, 1000])
            if set_position_ret:
                print("Motor set position successful!")
            else:
                print("Motor set position failed!")
            time.sleep(1)
            get_position_ret = hand.get_motor_real_angle()
            print(f'Get motor angle：{get_position_ret}')
        else:
            print("Device initialization failed!")
        close_ret = hand.close_device()
        if close_ret:
            print("Device close successful!")
        else:
            print("Device close failed!")
    else:
        print("Device open failed!")

- 结果输出：

- Device open successful!

- 发送数据: 01 04 00 00 00 01 31 CA

- 接收数据: 01 04 02 00 01 78 F0

- Device initialization successful!

- 发送数据: 01 10 00 1D 00 06 0C 00 64 00 64 00 64 00 64 00 64 00 64 A6 47

- 接收数据: 01 10 00 1D 00 06 D0 0D

- Motor set speed successful!

- 发送数据: 01 06 00 29 00 01 99 C2

- 接收数据: 01 06 00 29 00 01 99 C2

- Motor calibration successful!

- 发送数据: 01 10 00 0B 00 06 0C 01 F4 00 00 03 E8 03 E8 03 E8 03 E8 6E E1

- 接收数据: 01 10 00 0B 00 06 31 C9

- Motor set position successful!

- 发送数据: 01 04 00 17 00 06 C0 0C

- 接收数据: 01 04 0C 00 00 01 F4 03 E8 03 E8 03 E8 03 E8 95 B7

- Get motor angle：[0, 500, 1000, 1000, 1000, 1000]

- Device close successful!

- Set the full hand joint motor PVT control mode of ZWHAND

- from DOF6_ZWHAND_API import ZWHAND


if __name__ == '__main__':
    hand = ZWHAND('COM1', 115200, 0x01)
    open_ret = hand.open_start_device()
    if open_ret:
        print("Device open successful!")
        init_state = hand.get_initialize_state()
        if init_state:
            print("Device initialization successful!")
            calibration_ret = hand.set_all_motor_calibration()
            if calibration_ret:
                print("Motor calibration successful!")
            else:
                print("Motor calibration failed!")
            time.sleep(20)
            set_mode_ret = hand.set_finger_control_mode(0x02)
            if set_mode_ret:
                print("Set finger control mode successful!")
                set_pvt_ret = hand.set_pvt_control_param([400, 33000, 0] * 0x06)
                if set_pvt_ret:
                    print("Set pvt control param successful!")
                else:
                    print("Set pvt control param failed!")
                time.sleep(1)
                get_angle_ret = hand.get_motor_real_angle()
                if get_angle_ret:
                    print(f'Get motor angle：{get_angle_ret}')
                else:
                    print("Get motor real angle failed!")
            else:
                print("Set finger control mode failed!")
        else:
            print("Device initialization failed!")
        close_ret = hand.close_device()
        if close_ret:
            print("Device close successful!")
        else:
            print("Device close failed!")
    else:
        print("Device open failed!")

- 结果输出：

- Device open successful!

- 发送数据: 01 04 00 00 00 01 31 CA

- 接收数据: 01 04 02 00 01 78 F0

- Device initialization successful!

- 发送数据: 01 06 00 29 00 01 99 C2

- 接收数据: 01 06 00 29 00 01 99 C2

- Motor calibration successful!

- 发送数据: 01 10 00 31 00 06 0C 00 02 00 02 00 02 00 02 00 02 00 02 B7 8A

- 接收数据: 01 10 00 31 00 06 11 C4

- Set finger control mode successful!

- 发送数据: 01 10 00 37 00 12 24 01 90 80 E8 00 00 01 90 80 E8 00 00 01 90 80 E8 00 00 01 90 80 E8 00 00 01 90 80 E8 00 00 01 90 80 E8 00 00 FA 49

- 接收数据: 01 10 00 37 00 12 F1 CA

- Set pvt control param successful!

- 发送数据: 01 04 00 17 00 06 C0 0C

- 接收数据: 01 04 0C 01 90 01 90 01 90 01 90 01 90 01 90 95 B7

- Get motor angle：[400, 400, 400, 400, 400, 400]

- Device close successful!

- Set the full hand joint motor PC control mode of ZWHAND

- from DOF6_ZWHAND_API import ZWHAND


if __name__ == '__main__':
    hand = ZWHAND('COM1', 115200, 0x01)
    open_ret = hand.open_start_device()
    if open_ret:
        print("Device open successful!")
        init_state = hand.get_initialize_state()
        if init_state:
            print("Device initialization successful!")
            calibration_ret = hand.set_all_motor_calibration()
            if calibration_ret:
                print("Motor calibration successful!")
            else:
                print("Motor calibration failed!")
            time.sleep(20)
            set_mode_ret = hand.set_finger_control_mode(0x00)
            if set_mode_ret:
                print("Set finger control mode successful!")
                set_pvt_ret = hand.set_pc_control_param([0, -5000, 5000, 6000, 7000, 8000])
                if set_pvt_ret:
                    print("Set pc control param successful!")
                else:
                    print("Set pc control param failed!")
                time.sleep(1)
                get_angle_ret = hand.get_motor_real_angle()
                if get_angle_ret:
                    print(f'Get motor angle：{get_angle_ret}')
                else:
                    print("Get motor real angle failed!")
            else:
                print("Set finger control mode failed!")
        else:
            print("Device initialization failed!")
        close_ret = hand.close_device()
        if close_ret:
            print("Device close successful!")
        else:
            print("Device close failed!")
    else:
        print("Device open failed!")

- 结果输出：

- Device open successful!

- 发送数据: 01 04 00 00 00 01 31 CA

- 接收数据: 01 04 02 00 01 78 F0

- Device initialization successful!

- 发送数据: 01 06 00 29 00 01 99 C2

- 接收数据: 01 06 00 29 00 01 99 C2

- Motor calibration successful!

- 发送数据: 01 10 00 31 00 06 0C 00 00 00 00 00 00 00 00 00 00 00 00 DF 53

- 接收数据: 01 10 00 31 00 06 11 C4

- Set finger control mode successful!

- 发送数据: 01 10 00 49 00 06 0C 00 00 EC 78 13 88 17 70 1B 58 1F 40 BE 10

- 接收数据: 01 10 00 49 00 06 91 DD

- Set pc control param successful!

- 发送数据: 01 04 00 17 00 06 C0 0C

- 接收数据: 01 04 0C 00 00 00 00 03 E8 03 E8 03 E8 03 E8 95 B7

- Get motor angle：[0, 0, 1000, 1000, 1000, 1000]

- Device close successful!